Name: Ashish kumar sahni

Assignment Module -13: Windows Networking Services

- Discuss the role of Windows Firewall in Windows Server and how to configure it.

Ans: Windows firewall protects your server by managing what traffic may enter or leave. It rejects unwanted connections and only allows incoming ones you have approved such as specific apps, ports, or Ips. It’s a simple but essential security layers.

Here we configure using the GUI interface:

First open server manager>tool>windows defender firewall with advanced security.

Second go to inbound/outbound rules.

Third create a new policy to enable or disable things such as ports or programs.

Also, with the help of POWERSHELL using CLI based:

First turn on the firewall

Set -NetFirewallProfile-Profile Domain,Private,Public -Enable True

Second for allow a port like 8080:

New-NetfirewallRule -DisplayName “Allow HTTP” -Direction Inbound -Protocol TCP -Localport 8080 -Action Allow

Open Group policy Management:

Navigate to Computer Config>Windows Settings>Security Settings>Windows Defender Firewall.

Enable NAT: OPEN RRAS from Tools.

Right-click your server-> Configure and enable routing and remote access.

Select NAT setup> configure your internet-facing adapter as public.

Set the LAN adapter as private

All set!

- What is Network Address Translation (NAT) in Windows Server, and how do you configure it?

Ans: NAT (Network Address Translation) allows multiple devices on a private network to use a single public IP to connect to the internet. Its helpful for internet sharing and concealing internal IPs.

There are some set up NAT to quick steps to install RRAS:

Server Manger>Add Roles and features->Select Remote Access>Routing.

3. Explain the concept of Dynamic Host Configuration Protocol (DHCP) and how to configure it in Windows Server 2016.

Ans: DHCP (Dynamic Host Configuration Protocol) is a service that automatically assigns IP addresses to computers on a network. Rather than manually configuring Ips, the DHCP server configures them for you. It also distributes other information such as the gateway and DNS.

Setting Up DHCP on Windows server 2016:

- Install DHCP role > open server manager> click manage> add roles features.

Select DHCP server and install it. After installation, click complete DHCP configuration and authorize server.

- Create a Scope> Open DHCP from server manager> tool

Right-click on IPv4-> New Scope.

Set a name and add:

Start and end IP address range

Subnet mask

Exclusions (Ips that you do not want to distribute)

Lease time.

Set your Gateway and DNS

Then turn on the scope.

- Test It on client PC, ensure it is set to get IP automatically.

Execute ipconfig /release followed by ipconfig /renew in Command Prompt to obtain a new IP.

4. Describe the process of configuring DNS (Domain Name System) in Windows Server.

Ans: DNS convert names such as youtube.com into the IP addresses that computers use to communicate with one another. It’s a kind of phonebook for the web.

These are the process to configure DNS on windows server

- Install DNS role> Open server manger> click manage> add roles and features.

Choose DNS server, then install it.

When its completed, open DNS from the tools menu.

- Create a DNS zone in the DNS manager, right-click forward lookup zones-> select New Zone.

Select primary zone.

Enter your domain name (such as mycompany.local).

Complete the wizard with default settings.

- Add records Right-click your new zone> click new Host(A).

Enter a name (such as pc1) and IP address such as 192.168.1.10

Click add host.

Now pc1.mycompany.local resolves to that IP.

- (Optional) add reverse lookup zone Right-click Reverse lookup zones> new zone.

Use your network ID for example 192.168.1 to allow DNS to translate Ips to names.

- Inform Devices to use this DNS on every computer or via DHCP, set the DNS server to your Windows servers IP.

5.What is Server Manager, and how do you use it to manage servers in Windows Server?

Ans: Server Manager is a feature in windows server that makes it easy for you to mange your server. Install roles such as DNS or DHCP, monitor system health, and manage other servers-all form one location.

What can you do with it?

Add/remove server roles and features

View alerts and performance information

Manage many servers from one screen

Launch tools such as DNS Manager, DHCP, Group policy, etc.

How to use it?

- Open server manager automatically opens or search it in start menu.

- Press “manage” in order to install roles/features.

- Utilize “tools” for other server tools.

- Add additional servers in “manage”> “Add servers”.

6. Discuss the role of Remote Desktop Services (RDS) in Windows Server 2016 or 2019 and how to configure it.

Ans: RDS allows users to log into a windows server from another PC to access the full desktop or simply individual apps. It is similar to having a PC under you, except it’s the server that performs the tasks instead.

It uses are ability to work from remote locations.

No app needs to be installed on each individual computer

Good for shared computers or centralized applications

Lets quick version > open server manger > manage> features.

Select remote desktop services, select quick start.

Select session-based desktop deployment

Complete the wizard and allow it to install then add your users to remote desktop users

Configure licensing if necessary allow users to connect using remote desktop connection.